Task-1

A bit stream 1101011011 is transmitted using the standard CRC method. The generator polynomial is x4+x+1.

- What is the actual bit string transmitted?

To calculate the transmitted bit string using CRC:

- Augment the bitstream:
The generator polynomial has a degree of 4, so append 4 zeros to the original bitstream 1101011011.
Augmented bitstream = 11010110110000.

- Perform binary division (XOR):
Divide the augmented bitstream 11010110110000 by the generator polynomial 10101.
After performing the division (XOR), the remainder (CRC) is 1000.

- Transmit the original bitstream plus the CRC:
The transmitted bit string is the original bitstream 1101011011 concatenated with the CRC 1000.
Transmitted bit string: 11010110111000

- Suppose the third bit from the left is inverted during transmission. How will receiver detect this error?

Received bitstream with error:
If the third bit is inverted, the received bitstream becomes 11110110111000.

Receiver performs binary division:
The receiver will perform the same division (XOR) on the received bitstream 11110110111000 using the generator polynomial 10101.
Since the remainder after division is non-zero, the receiver detects that an error has occurred.

Task-2

A bit stream 10011101 is transmitted using the standard CRC method. The generator polynomial is x3+1.

- What is the actual bit string transmitted?

To calculate the transmitted bit string using CRC:

- Augment the bitstream:
The generator polynomial has a degree of 3, so append 3 zeros to the original bitstream 10011101.
Augmented bitstream = 100111010000.

- Perform binary division (XOR):
Divide the augmented bitstream 100111010000 by the generator polynomial 1001.
After performing the division (XOR), the remainder (CRC) is 000.

- Transmit the original bitstream plus the CRC:
The transmitted bit string is the original bitstream 10011101 concatenated with the CRC 000.
Transmitted bit string: 10011101000

- Suppose the third bit from the left is inverted during transmission. How will receiver detect this error?

Received bitstream with error:
If the third bit is inverted, the received bitstream becomes 10111101000.

Receiver performs binary division:
The receiver will perform the same division (XOR) on the received bitstream 10111101000 using the generator polynomial 1001.
In this case, the remainder after division is zero, meaning the receiver will not detect the error. This is because the error does not affect the CRC check for this particular generator polynomial.